MATERIAL SAFETY DATA SHEET

Product: PRESERVATIVE OIL 1009 H

Date: 05/10/2026

## 1. IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

Product identifier: PRESERVATIVE OIL 1009 H

CAS number: Not available

Use of substance/mixture: Water-based preservative oil for wooden product surfaces

Supplier: Binh Thanh Chemical Glue Production & Trading Co., Ltd (BT Chem)

Address: Group 4, Binh Phuoc B Quarter, An Phu Ward, Ho Chi Minh City, Vietnam

Phone: +84 274 3789 99 (1-7)

Email: lab@btchem.com.vn

Emergency phone: +84 274 3789 993

## 2. HAZARDS IDENTIFICATION

GHS Classification: Not classified as hazardous under GHS criteria for this product.

Label elements:

Hazard pictograms: Product does not require a hazard warning label under GHS criteria.

GHS label: Product does not require a hazard warning label under GHS criteria.

Hazard statements: Not applicable

Precautionary statements: Not applicable

Other hazards: Not applicable

## 3. COMPOSITION/INFORMATION ON INGREDIENTS

| Component | CAS No. | % by weight |
| --- | --- | --- |
| Water-based polymer resin (aqueous dispersion) | Not available (mixture) | 60 – 70 |
| Glycerol | 56-81-5 | 10 – 15 |
| Deionised water | 7732-18-5 | 5 – 10 |
| Monoethylene glycol | 107-21-1 | 5 – 10 |
| Amorphous silica (matting agent) | 7631-86-9 | 2 – 3 |
| Additives (defoamer, wetting agent, surface additive, preservative, rheology modifier) | Not available | < 2 |

## 4. FIRST AID MEASURES

Eye contact: Rinse cautiously with water for several minutes. Remove contact lenses if present and easy to do. Continue rinsing. Seek medical attention if pain persists.

Skin contact: Wash with plenty of soap and water. If skin irritation or rash occurs: seek medical advice. Wash contaminated clothing before reuse.

Inhalation: If symptoms appear: move to fresh air and ventilate the suspected area.

Ingestion: Do NOT induce vomiting. If vomiting occurs spontaneously, keep head lower than hips. Transport to nearest medical facility.

## 5. FIREFIGHTING MEASURES

Suitable extinguishing media: Dry sand, CO₂, foam, dry powder.

Unsuitable extinguishing media: Strong water jet.

Hazardous combustion products: CO₂, CO, toxic pyrolysis products.

Firefighter protection: Use breathing apparatus, protective clothing, rubber boots, gloves.

## 6. ACCIDENTAL RELEASE MEASURES

Personal precautions: See section 8 for PPE. Wear protective clothing and gloves.

Emergency procedures: Absorb with inert absorbent material (sand, sawdust, silica gel).

Environmental precautions: Avoid discharge into the environment.

Cleanup methods: Clean up spills immediately using absorbent material. Collect waste in labeled containers per local regulations.

## 7. HANDLING AND STORAGE

Handling: Wash hands after handling. Ensure good ventilation. Comply with occupational exposure limits.

Storage: Cool, dry, well-ventilated area. Keep container closed. Keep away from heat/sunlight. Storage temperature: 5°C–35°C.

## 8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Exposure limits: Not established.

Respiratory protection: Use organic vapor respirator if inhalation risk exists.

Eye protection: Wear splash goggles. Eyewash station nearby.

Skin protection: Wear impermeable gloves, protective clothing.

General: No eating/drinking/smoking at workplace.

## 9. PHYSICAL AND CHEMICAL PROPERTIES

| Property | Value |
| --- | --- |
| Physical state | Liquid |
| Color | Milky white |
| Odor | Mild characteristic odor |
| pH | No data |
| Melting point | No data |
| Boiling point | No data |
| Flammability | Not flammable |
| Vapor pressure | No data |
| Relative density | No data |
| Solubility | Dispersible in water |
| Viscosity | No data |

## 10. STABILITY AND REACTIVITY

Stability: Stable under normal conditions and recommended storage.

Conditions to avoid: Heat and open flames.

Materials to avoid: Strong oxidizing agents, strong alkalis, strong acids.

Hazardous decomposition: CO, CO₂, and other organic compounds upon combustion or thermal decomposition.

## 11. TOXICOLOGICAL INFORMATION

| Endpoint | Classification |
| --- | --- |
| Acute skin irritation | Not classified |
| Acute eye irritation | Not classified |
| Respiratory sensitization | Not classified |

## 12. ECOLOGICAL INFORMATION

Not classified as hazardous to the environment.

| Parameter | Value |
| --- | --- |
| Ecotoxicity | None |
| Bioaccumulation | None |
| Persistence/degradability | None |

## 13. DISPOSAL CONSIDERATIONS

Dispose in accordance with official regulations. Do not pour into drains.

## 14. TRANSPORT INFORMATION

Domestic/Sea/Air: Not classified as dangerous goods under transport regulations.

## 15. REGULATORY INFORMATION

EU Regulations: Does not contain restricted REACH Annex XVII substances, REACH Candidate List substances, or REACH Annex XIV substances.

Product does not contain toxic heavy metals, free formaldehyde, azo compounds, phthalates, benzene. Complies with REACH, EN.71-3 (Europe), and CPSIA, ASTM (USA).

## 16. OTHER INFORMATION

Preparation date: 05/10/2026

The information describes safety requirements based on current knowledge. It is intended for safe handling, storage, processing, transport, and disposal of the product. Information does not necessarily apply to mixtures or processed materials.